# Servo and Rain sensor

Sensor pin -> 4

Servo pin -> 5

#include <Servo.h>

#include <Servo.h>

Servo tap_servo;

int sensor_pin = 4;

int tap_servo_pin =5;

int val;

void setup(){

pinMode(sensor_pin,INPUT);

tap_servo.attach(tap_servo_pin);

}

void loop(){

val = digitalRead(sensor_pin);

if (val==0)

{tap_servo.write(0);

}

if (val==1)

{tap_servo.write(180);

}

}

# DHT22

//* How to use the DHT-22 sensor with Arduino

//   Temperature and humidity sensor and

//   I2C LCD1602

//  SDA --> A4

//  SCL --> A5

//DHT 22 out-->D7

//Libraries

#include <DHT.h>;

//I2C LCD:

#include <LiquidCrystal_I2C.h>

#include <Wire.h>

LiquidCrystal_I2C lcd(0x27,16,2); // set the LCD address to 0x27 for a 16 chars and 2 line display,Pls check your lcd.

//Constants

#define DHTPIN 7     // what pin we're connected to

#define DHTTYPE DHT22   // DHT 22

DHT dht(DHTPIN, DHTTYPE); //// Initialize DHT sensor for normal 16mhz Arduino

//Variables

//int chk;

int h;  //Stores humidity value

int t; //Stores temperature value

void setup()

{

Serial.begin(9600);

Serial.println("Temperature and Humidity Sensor Test");

dht.begin();

lcd.init(); //initialize the lcd

lcd.backlight(); //open the backlight

}

void loop()

{

//Read data and store it to variables h (humidity) and t (temperature)

// Reading temperature or humidity takes about 250 milliseconds!

h = dht.readHumidity();

t = dht.readTemperature();

//Print temp and humidity values to serial monitor

Serial.print("Humidity: ");

Serial.print(h);

Serial.print(" %, Temp: ");

Serial.print(t);

Serial.println(" ° Celsius");

// set the cursor to (0,0):

// print from 0 to 9:

lcd.setCursor(0, 0);

lcd.println(" Now Temperature ");

lcd.setCursor(0, 1);

lcd.print("T:");

lcd.print(t);

lcd.print("C");

lcd.setCursor(6, 1);

lcd.println("2021 ");

lcd.setCursor(11, 1);

lcd.print("H:");

lcd.print(h);

lcd.print("%");

delay(1000); //Delay 1 sec.

}

# BMP180

A4 -> LCD & sensor’s SDA

A5 -> LCD & sensor’s SCL

3.3v -> LCD & sensor’s VCC

GND -> LCD & sensor’s GND

#include <Wire.h>

#include <Adafruit_Sensor.h>

#include <Adafruit_BMP085_U.h>

#include <LiquidCrystal_I2C.h>

// Initialize the BMP180 sensor

Adafruit_BMP085_Unified bmp = Adafruit_BMP085_Unified(180); // '180' is just an ID for the sensor

// Initialize the LCD with I2C address 0x27 (adjust based on your LCD)

LiquidCrystal_I2C lcd(0x27, 16, 2); // 16x2 LCD

void setup() {

Serial.begin(9600);

Serial.println("BMP180 Sensor Test");

// Initialize the LCD

lcd.init();            // Initialize the LCD

lcd.backlight();       // Turn on the backlight

// Initialize the BMP180 sensor

if (!bmp.begin()) {

Serial.println("Could not find a valid BMP180 sensor, check wiring!");

lcd.setCursor(0, 0);

lcd.print("BMP180 not found");

while (1); // Halt if sensor is not found

}

// Display a startup message on the LCD

lcd.setCursor(0, 0);

lcd.print("BMP180 Sensor");

lcd.setCursor(0, 1);

lcd.print("Initializing...");

delay(2000); // Wait for 2 seconds

lcd.clear();

}

void loop() {

sensors_event_t event;

bmp.getEvent(&event);

if (event.pressure) {

// Get pressure

float pressure = event.pressure;

bmp.getTemperature(&temperature);

// Get temperature

float temperature;

bmp.getTemperature(&temperature);

// Calculate altitude

float altitude = bmp.pressureToAltitude(1013.25, pressure);

// Print readings to Serial Monitor

Serial.print("Pressure: ");

Serial.print(pressure);

Serial.println(" hPa");

Serial.print("Temperature: ");

Serial.print(temperature);

Serial.println(" °C");

Serial.print("Altitude: ");

Serial.print(altitude);

Serial.println(" m");

// Display readings on the LCD

lcd.setCursor(0, 0);

lcd.print("T: ");

lcd.print(temperature, 1); // 1 decimal place

lcd.print("C ");

lcd.print("P: ");

lcd.print(pressure, 1); // 1 decimal place

lcd.setCursor(0, 1);

lcd.print("Alt: ");

lcd.print(altitude, 2); // 1 decimal place

lcd.print(" m");

} else {

// If the sensor fails, display an error

Serial.println("Sensor error!");

lcd.setCursor(0, 0);

lcd.print("Sensor Error!");

}

delay(2000); // Wait for 2 seconds before the next reading

}

#### 1. Connect the SD Card Module

- Connect the SD card module to the Arduino:

  - VCC → 5V

  - GND → GND

  - MOSI → Pin 11

  - MISO → Pin 12

  - SCK → Pin 13

  - CS → Pin 10 (or any digital pin you define)

#### 2. Connect the Sensor

- Attach the desired sensor to the appropriate Arduino pins. For example:

  - For a DHT11 temperature and humidity sensor:

    - VCC → 5V

    - GND → GND

    - DATA → Pin 2

#include <SD.h>

#include <DHT.h>

#define CS_PIN 10        // Chip Select Pin for SD Module

#define DHT_PIN 2        // Data Pin for DHT Sensor

#define DHT_TYPE DHT11   // DHT 11

DHT dht(DHT_PIN, DHT_TYPE);

void setup() {

Serial.begin(9600);

dht.begin();

// Initialize SD Card

if (!SD.begin(CS_PIN)) {

Serial.println("SD card initialization failed!");

while (true);

}

Serial.println("SD card initialized.");

// Create/Open a file

File dataFile = SD.open("datalog.txt", FILE_WRITE);

if (dataFile) {

dataFile.println("Time, Temperature (°C), Humidity (%)");

dataFile.close();

}

}

void loop() {

float temperature = dht.readTemperature();

float humidity = dht.readHumidity();

// Check for valid sensor readings

if (isnan(temperature) || isnan(humidity)) {

Serial.println("Failed to read from DHT sensor!");

return;

}

// Print to Serial Monitor

Serial.print("Temperature: ");

Serial.print(temperature);

Serial.print(" °C | Humidity: ");

Serial.print(humidity);

Serial.println(" %");

// Log data to SD card

File dataFile = SD.open("datalog.txt", FILE_WRITE);

if (dataFile) {

dataFile.print(millis());

dataFile.print(", ");

dataFile.print(temperature);

dataFile.print(", ");

dataFile.println(humidity);

dataFile.close();

} else {

Serial.println("Error opening datalog.txt!");

}

delay(2000); // Log data every 2 seconds

}

#include <Servo.h>

#include <DHT.h>

#include <LiquidCrystal_I2C.h>

#include <Wire.h>

// Servo

Servo tap_servo;

// Sensor pins

int sensor_pin = 4;

int tap_servo_pin = 5;

// Variables

int val;

// Constants for DHT

#define DHTPIN 7     // Pin connected to the DHT sensor

#define DHTTYPE DHT22

DHT dht(DHTPIN, DHTTYPE);

// LCD setup

LiquidCrystal_I2C lcd(0x27, 16, 2); // Address 0x27, 16 chars, 2 lines

// Variables for temperature and humidity

float h;  // Stores humidity value

float t;  // Stores temperature value

void setup() {

// Initialize Serial Monitor

Serial.begin(9600);

Serial.println("Temperature and Humidity Sensor Test");

// Initialize DHT sensor

dht.begin();

// Initialize LCD

lcd.init();

lcd.backlight();

// Initialize Servo

tap_servo.attach(tap_servo_pin);

// Configure sensor pin

pinMode(sensor_pin, INPUT);

}

void loop() {

// Read the digital sensor

val = digitalRead(sensor_pin);

// Control the servo based on sensor input

if (val == 0) {

tap_servo.write(0);

} else if (val == 1) {

tap_servo.write(180);

}

// Read humidity and temperature

h = dht.readHumidity();

t = dht.readTemperature();

// Check if the reading is valid

if (isnan(h) || isnan(t)) {

Serial.println("Failed to read from DHT sensor!");

return;

}

// Print temp and humidity values to Serial Monitor

Serial.print("Humidity: ");

Serial.print(h);

Serial.print(" %, Temp: ");

Serial.print(t);

Serial.println(" °C");

// Display on LCD

lcd.setCursor(0, 0);

lcd.print("Temp: ");

lcd.print(t);

lcd.print("C");

lcd.setCursor(0, 1);

lcd.print("Hum: ");

lcd.print(h);

lcd.print("%");

delay(1000); // Delay for 1 second

}

#include <Wire.h>

#include <Adafruit_Sensor.h>

#include <Adafruit_BMP085_U.h>

#include <LiquidCrystal_I2C.h>

#include <Servo.h>

#include <DHT.h>

// BMP180 Sensor Initialization

Adafruit_BMP085_Unified bmp = Adafruit_BMP085_Unified(180); // '180' is just an ID for the sensor

// LCD Initialization

LiquidCrystal_I2C lcd(0x27, 16, 2); // 16x2 LCD at address 0x27

// DHT Sensor Initialization

#define DHTPIN 7     // Pin connected to the DHT sensor

#define DHTTYPE DHT22

DHT dht(DHTPIN, DHTTYPE);

// Servo Initialization

Servo tap_servo;

// Sensor and Servo Pins

int sensor_pin = 4;       // Digital input sensor pin

int tap_servo_pin = 5;    // Servo control pin

// Variables

int val;                  // Digital sensor reading

float h;                  // Humidity

float dht_temp;           // Temperature from DHT22

float bmp_temp;           // Temperature from BMP180

float pressure;           // Pressure

float altitude;           // Altitude

void setup() {

// Serial Monitor Initialization

Serial.begin(9600);

Serial.println("System Initialization");

// Initialize BMP180 Sensor

if (!bmp.begin()) {

Serial.println("Could not find a valid BMP180 sensor, check wiring!");

lcd.setCursor(0, 0);

lcd.print("BMP180 not found");

while (1); // Halt if sensor is not found

}

// Initialize DHT Sensor

dht.begin();

// Initialize LCD

lcd.init();

lcd.backlight();

// Initialize Servo

tap_servo.attach(tap_servo_pin);

// Configure Sensor Pin

pinMode(sensor_pin, INPUT);

// Startup Message

lcd.setCursor(0, 0);

lcd.print("System Ready");

lcd.setCursor(0, 1);

lcd.print("Initializing...");

delay(2000);

lcd.clear();

}

void loop() {

// Read the digital sensor

val = digitalRead(sensor_pin);

// Control the servo based on sensor input

if (val == 0) {

tap_servo.write(0);

} else if (val == 1) {

tap_servo.write(180);

}

// Read humidity and temperature from DHT

h = dht.readHumidity();

dht_temp = dht.readTemperature();

// Check if the DHT readings are valid

if (isnan(h) || isnan(dht_temp)) {

Serial.println("Failed to read from DHT sensor!");

lcd.setCursor(0, 0);

lcd.print("DHT Error");

return;

}

// Read pressure and temperature from BMP180

sensors_event_t event;

bmp.getEvent(&event);

if (event.pressure) {

pressure = event.pressure;

bmp.getTemperature(&bmp_temp); // Store BMP180 temperature reading

altitude = bmp.pressureToAltitude(1013.25, pressure); // Calculate altitude

} else {

Serial.println("BMP180 sensor error!");

lcd.setCursor(0, 0);

lcd.print("BMP180 Error");

return;

}

// Print sensor readings to Serial Monitor

Serial.print("DHT Temp: ");

Serial.print(dht_temp);

Serial.println(" °C");

Serial.print("BMP Temp: ");

Serial.print(bmp_temp);

Serial.println(" °C");

Serial.print("Humidity: ");

Serial.print(h);

Serial.println(" %");

Serial.print("Pressure: ");

Serial.print(pressure);

Serial.println(" hPa");

Serial.print("Altitude: ");

Serial.print(altitude);

Serial.println(" m");

// Display readings on the LCD

//lcd.setCursor(0, 0);

//lcd.print("DHT: ");

//lcd.print(dht_temp, 1); // 1 decimal place

//lcd.print("C");

lcd.setCursor(0, 0);

lcd.print("T:");

lcd.print(bmp_temp, 2); // 1 decimal place

lcd.print("C");

lcd.print(" H:");

lcd.print(h, 1); // 1 decimal place

lcd.print("%");

lcd.setCursor(0, 1);

lcd.print("P: ");

lcd.print(pressure, 2); // 1 decimal place

lcd.print("hPa");

delay(2000); // Delay for 2 seconds

}

#include <Wire.h>

#include <Adafruit_Sensor.h>

#include <Adafruit_BMP085_U.h>

#include <LiquidCrystal_I2C.h>

#include <Servo.h>

#include <DHT.h>

#include <SD.h>

// BMP180 Sensor Initialization

Adafruit_BMP085_Unified bmp = Adafruit_BMP085_Unified(180);

// LCD Initialization

LiquidCrystal_I2C lcd(0x27, 16, 2);

// SD Card Pin

#define CS_PIN 10

// DHT Sensor Initialization

#define DHTPIN 7

#define DHTTYPE DHT22

DHT dht(DHTPIN, DHTTYPE);

// Servo Initialization

Servo tap_servo;

// Pins

int sensor_pin = 4;

int tap_servo_pin = 5;

// Variables

int val;

float h, dht_temp, bmp_temp, pressure, altitude;

void setup() {

Serial.begin(9600);

Serial.println("System Initialization");

// Initialize BMP180

if (!bmp.begin()) {

Serial.println("Could not find BMP180!");

lcd.init();

lcd.backlight();

lcd.setCursor(0, 0);

lcd.print("BMP180 Error");

while (1);

}

// Initialize DHT Sensor

dht.begin();

// Initialize LCD

lcd.init();

lcd.backlight();

lcd.setCursor(0, 0);

lcd.print("System Ready");

delay(2000);

lcd.clear();

// Initialize Servo

tap_servo.attach(tap_servo_pin);

pinMode(sensor_pin, INPUT);

// Initialize SD Card

if (!SD.begin(CS_PIN)) {

Serial.println("SD card initialization failed!");

lcd.setCursor(0, 0);

lcd.print("SD Error");

while (1);

}

Serial.println("SD card initialized.");

// Create/Open a file

File dataFile = SD.open("datalog.txt", FILE_WRITE);

if (dataFile) {

dataFile.println("Time, Temperature (°C), Humidity (%), Pressure (hPa)");

dataFile.close();

}

// Reinitialize LCD after SD card

lcd.init();

lcd.backlight();

}

void loop() {

val = digitalRead(sensor_pin);

tap_servo.write(val == 0 ? 0 : 180);

// Read sensors

h = dht.readHumidity();

dht_temp = dht.readTemperature();

if (isnan(h) || isnan(dht_temp)) {

Serial.println("DHT Error");

lcd.setCursor(0, 0);

lcd.print("DHT Error");

return;

}

sensors_event_t event;

bmp.getEvent(&event);

if (event.pressure) {

pressure = event.pressure;

bmp.getTemperature(&bmp_temp);

altitude = bmp.pressureToAltitude(1013.25, pressure);

} else {

Serial.println("BMP180 Error");

lcd.setCursor(0, 0);

lcd.print("BMP180 Error");

return;

}

// Log data to SD card

File dataFile = SD.open("datalog.txt", FILE_WRITE);

if (dataFile) {

dataFile.print(millis());

dataFile.print(", ");

dataFile.print(bmp_temp);

dataFile.print(", ");

dataFile.print(h);

dataFile.print(", ");

dataFile.println(pressure);

dataFile.close();

} else {

Serial.println("SD Write Error");

}

// Print sensor readings to Serial Monitor

Serial.print("DHT Temp: ");

Serial.print(dht_temp);

Serial.println(" °C");

Serial.print("BMP Temp: ");

Serial.print(bmp_temp);

Serial.println(" °C");

Serial.print("Humidity: ");

Serial.print(h);

Serial.println(" %");

Serial.print("Pressure: ");

Serial.print(pressure);

Serial.println(" hPa");

Serial.print("Altitude: ");

Serial.print(altitude);

Serial.println(" m");

// Display on LCD

lcd.setCursor(0, 0);

lcd.print("T:");

lcd.print(bmp_temp, 2);

lcd.print("CH:");

lcd.print(h, 2);

lcd.print("%");

lcd.setCursor(0, 1);

lcd.print("P:");

lcd.print(pressure, 2);

lcd.print("hPa");

delay(2000);

}